[Code of Federal Regulations]

[Title 24, Volume 4, Parts 700 to 1699]

[Revised as of April 1, 2000]

From the U.S. Government Printing Office via GPO Access

[CITE: 24CFR982.401]

TITLE 24--HOUSING AND URBAN DEVELOPMENT

CHAPTER IX--OFFICE OF ASSISTANT SECRETARY FOR PUBLIC AND INDIAN HOUSING,

DEPARTMENT OF HOUSING AND URBAN DEVELOPMENT

PART 982--SECTION 8 TENANT BASED ASSISTANCE: HOUSING CHOICE VOUCHER PROGRAM--Table of Contents

Subpart I--Dwelling Unit: Housing Quality Standards, Subsidy Standards,

Inspection and Maintenance

Sec. 982.401  Housing quality standards (HQS).

Source: 60 FR 34695, July 3, 1995, unless otherwise noted.

(a) Performance and acceptability requirements. (1) This section

states the housing quality standards (HQS) for housing assisted in the

programs.

(2)(i) The HQS consist of:

(A) Performance requirements; and

(B) Acceptability criteria or HUD approved variations in the

acceptability criteria.

(ii) This section states performance and acceptability criteria for

these key aspects of housing quality:

(A) Sanitary facilities;

(B) Food preparation and refuse disposal;

(C) Space and security;

(D) Thermal environment;

(E) Illumination and electricity;

(F) Structure and materials;

(G) Interior air quality;

(H) Water supply;

(I) Lead-based paint;

(J) Access;

(K) Site and neighborhood;

(L) Sanitary condition; and

(M) Smoke detectors.

(3) All program housing must meet the HQS performance requirements

both at commencement of assisted occupancy, and throughout the assisted

tenancy.

(4)(i) In addition to meeting HQS performance requirements, the

housing must meet the acceptability criteria stated in this section,

unless variations are approved by HUD.

(ii) HUD may approve acceptability criteria variations for the

following purposes:

(A) Variations which apply standards in local housing codes or other

codes adopted by the PHA; or

(B) Variations because of local climatic or geographic conditions.

(iii) Acceptability criteria variations may only be approved by HUD

pursuant to paragraph (a)(4)(ii) of this section if such variations

either:

(A) Meet or exceed the performance requirements; or

(B) Significantly expand affordable housing opportunities for

families assisted under the program.

(iv) HUD will not approve any acceptability criteria variation if

HUD believes that such variation is likely to adversely affect the

health or safety of participant families, or severely restrict housing

choice.

(b) Sanitary facilities--(1) Performance requirements. The dwelling

unit must include sanitary facilities located in the unit. The sanitary facilities must be in proper operating condition, and adequate for personal

cleanliness and the disposal of human waste. The sanitary facilities

must be usable in privacy.

(2) Acceptability criteria. (i) The bathroom must be located in a

separate private room and have a flush toilet in proper operating

condition.

(ii) The dwelling unit must have a fixed basin in proper operating

condition, with a sink trap and hot and cold running water.

(iii) The dwelling unit must have a shower or a tub in proper

operating condition with hot and cold running water.

(iv) The facilities must utilize an approvable public or private

disposal system (including a locally approvable septic system).

(c) Food preparation and refuse disposal--(1) Performance

requirement. (i) The dwelling unit must have suitable space and

equipment to store, prepare, and serve foods in a sanitary manner.

(ii) There must be adequate facilities and services for the sanitary

disposal of food wastes and refuse, including facilities for temporary

storage where necessary (e.g, garbage cans).

(2) Acceptability criteria. (i) The dwelling unit must have an oven,

and a stove or range, and a refrigerator of appropriate size for the

family. All of the equipment must be in proper operating condition. The

equipment may be supplied by either the owner or the family. A microwave

oven may be substituted for a tenant-supplied oven and stove or range. A

microwave oven may be substituted for an owner-supplied oven and stove

or range if the tenant agrees and microwave ovens are furnished instead

of an oven and stove or range to both subsidized and unsubsidized

tenants in the building or premises.

(ii) The dwelling unit must have a kitchen sink in proper operating

condition, with a sink trap and hot and cold running water. The sink

must drain into an approvable public or private system.

(iii) The dwelling unit must have space for the storage,

preparation, and serving of food.

(iv) There must be facilities and services for the sanitary disposal

of food waste and refuse, including temporary storage facilities where

necessary (e.g., garbage cans).

(d) Space and security--(1) Performance requirement. The dwelling

unit must provide adequate space and security for the family.

(2) Acceptability criteria. (i) At a minimum, the dwelling unit must

have a living room, a kitchen area, and a bathroom.

(ii) The dwelling unit must have at least one bedroom or living/

sleeping room for each two persons. Children of opposite sex, other than

very young children, may not be required to occupy the same bedroom or

living/sleeping room.

(iii) Dwelling unit windows that are accessible from the outside,

such as basement, first floor, and fire escape windows, must be lockable

(such as window units with sash pins or sash locks, and combination

windows with latches). Windows that are nailed shut are acceptable only

if these windows are not needed for ventilation or as an alternate exit

in case of fire.

(iv) The exterior doors of the dwelling unit must be lockable.

Exterior doors are doors by which someone can enter or exit the dwelling

unit.

(e) Thermal environment--(1) Performance requirement. The dwelling

unit must have and be capable of maintaining a thermal environment

healthy for the human body.

(2) Acceptability criteria. (i) There must be a safe system for

heating the dwelling unit (and a safe cooling system, where present).

The system must be in proper operating condition. The system must be

able to provide adequate heat (and cooling, if applicable), either

directly or indirectly, to each room, in order to assure a healthy

living environment appropriate to the climate.

(ii) The dwelling unit must not contain unvented room heaters that

burn gas, oil, or kerosene. Electric heaters are acceptable.

(f) Illumination and electricity--(1) Performance requirement. Each

room must have adequate natural or artificial illumination to permit

normal indoor activities and to support the health and safety of occupants. The dwelling unit must have sufficient electrical sources so occupants can use essential electrical appliances. The electrical fixtures and wiring must ensure safety from fire.

(2) Acceptability criteria. (i) There must be at least one window in

the living room and in each sleeping room.

(ii) The kitchen area and the bathroom must have a permanent ceiling

or wall light fixture in proper operating condition. The kitchen area

must also have at least one electrical outlet in proper operating

condition.

(iii) The living room and each bedroom must have at least two

electrical outlets in proper operating condition. Permanent overhead or

wall-mounted light fixtures may count as one of the required electrical

outlets.

(g) Structure and materials--(1) Performance requirement. The

dwelling unit must be structurally sound. The structure must not present

any threat to the health and safety of the occupants and must protect

the occupants from the environment.

(2) Acceptability criteria. (i) Ceilings, walls, and floors must not

have any serious defects such as severe bulging or leaning, large holes,

loose surface materials, severe buckling, missing parts, or other

serious damage.

(ii) The roof must be structurally sound and weathertight.

(iii) The exterior wall structure and surface must not have any

serious defects such as serious leaning, buckling, sagging, large holes,

or defects that may result in air infiltration or vermin infestation.

(iv) The condition and equipment of interior and exterior stairs,

halls, porches, walkways, etc., must not present a danger of tripping

and falling. For example, broken or missing steps or loose boards are

unacceptable.

(v) Elevators must be working and safe.

(h) Interior air quality--(1) Performance requirement. The dwelling

unit must be free of pollutants in the air at levels that threaten the

health of the occupants.

(2) Acceptability criteria. (i) The dwelling unit must be free from

dangerous levels of air pollution from carbon monoxide, sewer gas, fuel

gas, dust, and other harmful pollutants.

(ii) There must be adequate air circulation in the dwelling unit.

(iii) Bathroom areas must have one openable window or other adequate

exhaust ventilation.

(iv) Any room used for sleeping must have at least one window. If

the window is designed to be openable, the window must work.

(i) Water supply--(1) Performance requirement. The water supply must

be free from contamination.

(2) Acceptability criteria. The dwelling unit must be served by an

approvable public or private water supply that is sanitary and free from

contamination.

(j) Lead-based paint performance requirement. The Lead-Based Paint

Poisoning Prevention Act (42 U.S.C. 4821-4846), the Residential Lead-

Based Paint Hazard Reduction Act of 1992 (42 U.S.C. 4851-4856), and

implementing regulations at part 35, subparts A, B, M, and R of this

title apply to units assisted under this part.

(k) Access performance requirement. The dwelling unit must be able

to be used and maintained without unauthorized use of other private

properties. The building must provide an alternate means of exit in case

of fire (such as fire stairs or egress through windows).

(l) Site and Neighborhood--(1) Performance requirement. The site and

neighborhood must be reasonably free from disturbing noises and

reverberations and other dangers to the health, safety, and general

welfare of the occupants.

(2) Acceptability criteria. The site and neighborhood may not be

subject to serious adverse environmental conditions, natural or manmade,

such as dangerous walks or steps; instability; flooding, poor drainage,

septic tank back-ups or sewage hazards; mudslides; abnormal air

pollution, smoke or dust; excessive noise, vibration or vehicular

traffic; excessive accumulations of trash; vermin or rodent infestation;

or fire hazards.

(m) Sanitary condition--(1) Performance requirement. The dwelling

unit and its equipment must be in sanitary condition.

(2) Acceptability criteria. The dwelling unit and its equipment must

be free of vermin and rodent infestation.

(n) Smoke detectors performance requirement--(1) Except as provided

in paragraph (n)(2) of this section, each dwelling unit must have at

least one battery-operated or hard-wired smoke detector, in proper

operating condition, on each level of the dwelling unit, including

basements but excepting crawl spaces and unfinished attics. Smoke

detectors must be installed in accordance with and meet the requirements

of the National Fire Protection Association Standard (NFPA) 74 (or its

successor standards). If the dwelling unit is occupied by any hearing-

impaired person, - smoke detectors must have an alarm system, designed

for hearing-impaired persons as specified in NFPA 74 (or successor

standards).

(2) For units assisted prior to April 24, 1993, owners who installed

battery-operated or hard-wired smoke detectors prior to April 24, 1993

in compliance with HUD's smoke detector requirements, including the

regulations published on July 30, 1992, (57 FR 33846), will not be

required subsequently to comply with any additional requirements

mandated by NFPA 74 (i.e., the owner would not be required to install a

smoke detector in a basement not used for living purposes, nor would the

owner be required to change the location of the smoke detectors that

have already been installed on the other floors of the unit).

[60 FR 34695, July 3, 1995, as amended at 61 FR 27163, May 30, 1996; 63

FR 23861, Apr. 30, 1998; 64 FR 26646, May 14, 1999; 64 FR 49658, Sept.

14, 1999; 64 FR 50230, Sept. 15, 1999]